APPENDIX TRANSIT TRAFFIC SERVICE

TABLE OF CONTENTS

1.	INTRODUCTION	3

2.	DEFINITIONS	3

3.	RESPONSIBILITIES OF THE PARTIES	6

4.	ACCESSIBLE LETTERS	10

5.	TRANSIT TRAFFIC RATE APPLICATION	10

6.	TRANSIT TRAFFIC ROUTING	11

7.	DIRECT TRUNKING REQUIREMENTS	12

It is SBC's position that Transit Traffic Service is not

Subject to arbitration in this section 251/252 proceeding, because it is not a section 251/252 service.  This Appendix is presented and filed solely in the event that the Commission decides to arbitrate Transit Traffic Service issues over SBC's objection.

## Transit Traffic Service Appendix

1.0          Introduction

  - This Transit Traffic Service Appendix (“Appendix”) sets forth the rates, terms and conditions of SBC-13STATE’s Transit Traffic Service as a Transit Service Provider. SBC-13STATE’s Transit Traffic Service Appendix sets forth the provisions of interconnection services to other Telecommunications Service providers whereby the Telecommunications traffic does not originate with (or terminate to) the Transit Service Provider’s End User. Transit Traffic Service allows Level 3 to exchange traffic with a Third Party Terminating Carrier to which it is not directly interconnected.

  - This Appendix incorporates the provisions of a transiting arrangement as it relates to SBC-13STATE’s provision of Transit Traffic Service as a Transit Service Provider to interconnected Competitive Local Exchange Carriers (CLECs) or to interconnected Out of Exchange Local Exchange Carriers (OE LECs) (i.e., carriers that interconnect with SBC-13STATE’s network but operate and/or provide Telecommunications Services outside of SBC-13STATE’s incumbent local exchange area).

1.3       Transit Traffic Service is an optional non 251 / 252 service provided by SBC-13STATE to Level 3 where Level 3 is directly interconnected with an SBC-13STATE Tandem. SBC 13-STATE neither originates nor terminates Transit Traffic on its network, but acts only as an intermediary. For the purposes of this Appendix, Transit Traffic Service is a service that is limited to Section 251(b)(5) Traffic, ISP-Bound Traffic, IntraLATA Toll Traffic, and  800 IntraLATA Toll Traffic (as defined in Sections 2.0below) destined  to the End Users of a Third Party Terminating Carrier (i.e.,  Competitive Local Exchange Carriers, Incumbent Local Exchange Carriers,   Commercial Mobile Radio Service (CMRS) providers or Out-of Exchange Local Exchange Carriers, but specifically excluding Interexchange Carriers ) (“Third Party Terminating Carrier”),  and  is routed utilizing an SBC-13STATE tandem switch where an SBC-13STATE End User is neither the originating nor the terminating party.

- Definitions

  - “800 IntraLATA Toll Traffic” is defined as traffic that originates from Level 3’s End User that utilizes a dialing sequence that invokes toll-free, 800-like, service processing, that terminates to an End User served by a Third Party Terminating Carrier, whereby the Third Party Terminating Carrier is both the Section 251(b)(5) Traffic Provider and the IntraLATA toll provider (not sent through an IXC or an intermediary).  “800 IntraLATA Toll Traffic” includes but is not limited to calls placed to 800, 877, 888, (“8YY”) NPA Service Access Codes (SAC).

2.2        “Accessible Letters” is defined as correspondence used to communicate pertinent

information regarding  SBC-13STATE to the client / End User community.

2.3        “Central Office Switch” means a switch, including, but not limited to an End Office Switch and a Tandem Switch.

  - “Calling Party Number” or “CPN” is as defined in 47 C.F.R. § 64.1600(c) ("CPN").

  - “Connecticut Transit Traffic Service” means all Section 251(b)(5) Traffic, ISP-Bound Traffic, IntraLATA Toll Traffic and/or 800 IntraLATA Toll Traffic delivered via the Transit Traffic Service whereby SBC CONNECTICUT will compensate the terminating carrier for applicable Transit Traffic, carrying out the terms and conditions herein.

  - “End Office” or “End Office Switch” is an SBC-13STATE switch that directly terminates traffic to and receives traffic from End Users of local Exchange Services.

  - “Exchange Service” means Telephone Exchange Service as defined in the Act.

2.8    “ISP-Bound Traffic” is defined as traffic  exchanged between Level 3’s End User and an Internet Service Provider (ISP) served by a Third Party Terminating Carrier that:

- originates from Level 3’s End Users and terminates to an ISP served by a Third Party Terminating Carrier in the same  SBC-13STATE exchange area; or

- originates from Level 3’s end-users and terminates to an ISP served by a Third Party Terminating Carrier within different SBC-13STATE Exchanges or within an SBC-13STATE exchange and an independent ILEC exchange, that share a common mandatory local calling area, as defined in SBC-13STATE’s tariff, (e.g., mandatory Extended Area Service (EAS), mandatory Extended Local Calling Service (ELCS), or other like types of mandatory expanded local calling scopes).

2.9      “IntraLATA Toll Traffic” is defined as traffic exchanged between  Level 3’s End Users and the End Users of a Third Party Terminating Carrier which subtends an SBC-13STATE Tandem, whereby the Transit Traffic originates in one mandatory local calling area and terminates in a different mandatory local calling area but where both mandatory local calling areas are within the same LATA.  Such IntraLATA Toll Traffic must terminate to a Third Party Terminating Carrier’s End User, whereby the Third Party Terminating Carrier is both the Section 251(b)(5) Traffic Provider and the IntraLATA toll provider (not sent through an IXC or an intermediary). For purposes of this Appendix, traffic between Level 3’s End Users that subscribe to one-way or two-way Optional Extended Area Service (Optional EAS) and the End User of a Third Party Terminating Carrier that is within the SBC-13STATE local or mandatory exchanges that are covered by an Optional EAS Plan will be treated as IntraLATA Toll Traffic.

2.10    “Section 251(b)(5) Traffic” is defined as traffic exchanged between Level 3’s  end users and the end users of a Third Party telecommunications carrier, other than a CMRS provider, that terminates the call (Third Party Terminating Carrier) that:

- originates and terminates to such end-users in the same SBC-13STATE exchange area; or

- originates and terminates to such end-users within different SBC-13STATE Exchanges, or within an SBC-13STATE exchange and an independent ILEC exchange, that share a common mandatory local calling area, as defined in SBC-13STATE’s tariff, (e.g., mandatory Extended Area Service (EAS), mandatory Extended Local Calling Service (ELCS), or other like types of mandatory expanded local calling scopes); or

For Section 251(b)(5) Traffic exchanged between Level 3’s end users and the end users of a CMRS provider that terminates the call, such traffic shall originate and terminate within the same Major Trading Area (MTA) as defined in 47 CFR§ 24.202(a).

  - “Tandem” or “Tandem Switch” is an SBC-13STATE switch used to connect  Trunks between and among other Central Office Switches.

2.12      “Third Party Originating Carrier” means a Telecommunications Carrier (e.g., Competitive Local Exchange Carrier (CLEC), Incumbent Local Exchange Carrier (ILEC), Commercial Mobile Radio Service (CMRS) provider or Out-of Exchange Local Exchange Carrier (OELEC)) that originates Transit Traffic that transits SBC-13STATE’s network and is delivered to Level 3.

2.13   “Third Party Terminating Carrier” means a Telecommunications Carrier to which traffic is terminated when Level 3 uses SBC-13STATE’s Transit Traffic Service (e.g., Competitive Local Exchange Carrier (CLEC), Incumbent Local Exchange Carrier (ILEC), Commercial Mobile Radio Service (CMRS) provider or Out-of Exchange Local Exchange Carrier (OELEC)).

2.14  “Transit Service Provider” means SBC-13STATE when providing its Transit Traffic Service.

2.15    “Transit Traffic” means all Section 251(b)(5) Traffic, ISP-Bound

Traffic, IntraLATA Toll Traffic, CMRS provider-bound traffic and/or 800 IntraLATA Toll Traffic delivered via the Transit Traffic Service.

  - “Trunk” or “Trunk Group” means the switch port interface(s) and the communication path created to connect Level 3’s network with SBC-13STATE’s network for the purpose of interconnection pursuant to the Act.

3.0        Responsibilities of the Parties

3.1      The Parties agree that SBC-13STATE is not obligated under Sections 251 and 252 of the Act to provide Level 3 with SBC-13STATE’s Transit Traffic Service as a means for Level 3 to indirectly interconnect with Third Party Terminating Carriers.   Level 3 has the option of using the Transiting Traffic services provided by SBC-13STATE or any other Telecommunications Carrier that provides similar services.

3.2      	SBC-12STATE will provide Level 3 with SBC-12STATE’s Transit Traffic Service to deliver Section 251(b)(5) Traffic, ISP-Bound Traffic, IntraLATA Toll Traffic, CMRS provider-bound traffic and/or 800 IntraLATA Toll Traffic to all Third Party Terminating Carriers with whom SBC-12STATE is interconnected   in the LATA, or outside of the LATA when a LATA boundary waiver exists.

  - SBC CONNECTICUT will make its Connecticut Transit Traffic Service available to Level 3 for the purpose of completing Level 3 Transit Traffic calls as defined in Sections 2.0 above, and upon the terms and conditions set forth herein. In doing so, SBC CONNECTICUT will compensate the terminating carrier for applicable Transit Traffic as defined in Section 2.5. above.

    - In SBC CONNECTICUT, the Connecticut Transit Traffic Service Rate applies when Level 3 sends Local and IntraLATA Toll traffic to a Third Party Terminating Carrier network through SBC CONNECTICUT’s Tandem.  Level 3  is responsible for payment of the Connecticut Transit Traffic Service Rate.  The Connecticut Transit Traffic Service Rate is only applicable when calls do not terminate to an SBC CONNECTICUT End User.  The Connecticut Transit Traffic Service Rate is specified in Appendix Pricing.

    - As part of SBC CONNECTICUT’s Connecticut Transit Traffic Service, SBC CONNECTICUT will be solely responsible for compensating the Third Party Terminating Carrier for Level 3’s Transit Traffic that is terminated on a Third Party Terminating Carrier’s network.

      - Level 3  shall bill SBC CONNECTICUT for terminating Transit Traffic originated by other LECs or CLECs in accordance with the provisions of this Appendix applicable to SBC CONNECTICUT originated Transit Traffic.

    - Level 3   shall issue ASRs for dedicated one-way Connecticut Transit Traffic Service trunk groups in SBC CONNECTICUT.

3.3.3.1 One-way Trunk Groups for Transit Traffic Service in SBC CONNECTICUT, (Connecticut Transit Traffic trunk groups) can be established between Level 3’s switch and an SBC CONNECTICUT’s Tandem.  Level 3 is financially responsible for the transport facility cost. These Trunk Groups will utilize Signaling System 7 (SS7) or multi-frequency (MF) signaling protocol, with SS7 signaling preferred whenever possible.

3.4      A Transit Traffic Service rate applies to all traffic that originated on Level 3’s network, transits  SBC-12STATE’s network and terminates to a Third Party Terminating Carrier’s network. The Transiting rate is only applicable when calls do not originate with (or terminate to) an SBC-12STATE End User.

  - Level 3 shall route appropriate traffic (i.e., only traffic to End Offices that subtend that Tandem or Transit Traffic) to the respective SBC-12STATE Tandems on the Trunk Groups defined below. SBC-12STATE shall route appropriate traffic to Level 3 switches on the Trunk Groups defined below.

3.6     The rates that SBC-12STATE shall charge Level 3 for the Transit Traffic Service is outlined in Section 5.0, below.

3.7 	Level 3 has the sole obligation to enter into traffic compensation arrangements with Third Party Terminating Carriers prior to delivering traffic to SBC-12STATE for transiting to such Third Party Terminating Carriers. In no event will SBC-12STATE have any liability to Level 3 or any Third Party if  Level 3 fails to enter into such traffic compensation arrangements.  In the event Level 3 originates traffic that transits SBC-12STATE’s network to reach a Third Party Terminating Carrier with whom  Level 3 does not have a traffic compensation arrangement, then Level 3 will indemnify, defend and hold harmless SBC-12STATE against any and all Losses including, without limitation, charges levied by such Third Party Terminating Carrier.  The Third Party Terminating Carrier and SBC-12STATE will bill their respective charges directly to Level 3.  SBC-12STATE will not be required to function as a billing intermediary, e.g. clearinghouse. Under no circumstances will SBC-12STATE be required to pay any termination charges to the Third Party Terminating Carrier.

3.8    Level 3 shall not charge SBC-12STATE when SBC-12STATE provides Transit Traffic Service as the Transit Traffic Provider for calls terminated to Level 3.

3.9       Each Party to this Appendix will be responsible for the accuracy and quality of its

data submitted to the other Party.

  - Level 3 will be responsible for sending the Calling Party Number (CPN) for calls originating on its network and passed to the network of a Third Party Terminating Carrier from SBC-13STATE serving as the Transit Traffic Provider.   Where SBC-13STATE is providing the Transit Traffic Service as defined in Section 1.3 above, SBC-13STATE will pass the Calling Party Number (CPN), if it is received from Level 3.  If the CPN is not received from the Level 3, SBC-13STATE can not forward the CPN and Level 3 will indemnify, defend and hold harmless SBC-13STATE from any and all Losses arising out of the failure of any traffic transiting SBC-13STATE’s network to have CPN.

  - When SBC-13STATE, operating as a Transit Service Provider, transits  Section 251(b)(5) Traffic, ISP-Bound Traffic, IntraLATA Toll Traffic, CMRS provider-bound traffic and/or 800 IntraLATA Toll Traffic to Level 3 from a Third Party Originating Carrier,   SBC-13STATE agrees to pass the originating CPN information to Level 3 as provided by the Third Party Originating Carrier.

  - For all Transit Traffic as defined in Section 2.15 above, Level 3 shall provide CPN as defined in 47 C.F.R. § 64.1600(c) ("CPN") and shall not strip, alter, modify, add, delete, change, or incorrectly assign any CPN.  If SBC-13STATE or Third Party Terminating Carrier identifies improper, incorrect, or fraudulent use of local exchange services or identifies stripped, altered, modified, added, deleted, changed, and/or incorrectly assigned CPN, Level 3 agrees to cooperate to investigate and take corrective action.   If Level 3 is passing CPN but SBC-13STATE or Third Party Terminating Carrier is not properly receiving information, Level 3 will work cooperatively to correct the problem.

3.13    Level 3 shall provide all SS7 signaling information including, without limitation, charge number and originating line information ("OLI").  For terminating Feature Group D traffic (“FGD”), SBC-13STATE will pass all SS7 signaling information including, without limitation, CPN if it receives CPN from FGD carriers.  All privacy indicators will be honored.  Where available, network signaling information such as transit network selection ("TNS") parameter, carrier identification codes (“CIC”) (CCS platform) and CIC/OZZ information (non-SS7 environment) will be provided by Level 3 wherever such information is needed for call routing or billing.  The Parties will follow all OBF adopted standards pertaining to TNS and CIC/OZZ codes.

3.14  	The Third Party Originating Carrier is responsible for sending the CPN for calls originating on its network and passed to the network of Level 3 from SBC-13STATE serving as the Transit Traffic Provider.   Where SBC-13STATE is providing a Transit Traffic Service as defined in Section 1.3 above, SBC-13STATE will pass the Calling Party Number (CPN), if it is received from a Third Party Originating Carrier.  If the CPN is not received from the Third Party Originating Carrier, SBC-13STATE can not forward the CPN; therefore, Level 3 will indemnify, defend and hold harmless SBC-13STATE from any Losses. according to Section 3.7 above.  If SBC-13STATE or Level 3 identifies stripped, altered, modified, added, deleted, changed, and/or incorrectly assigned CPN from Third Party Originating Carrier, Level 3 agrees to cooperate to work with Third Party Originating Carrier to investigate and take corrective action. If Third Party Originating Carrier is passing CPN but SBC-13STATE or Level 3 is not properly receiving information, Level 3 will work cooperatively to correct the problem.

  - Level 3 agrees to seek terminating compensation directly from the Third Party  Originating Carrier.  SBC-13STATE, as the Transit Service Provider will not be obligated to pay for Transit Traffic as the default originator.

# 4.0	Accessible Letters

4.1	SBC-13STATE communicates official information to CLECs via its Accessible Letter notification process.

4.2	In the SBC-13STATE’s Accessible Letter notification will be via electronic mail (“e-mail”) distribution. Accessible Letter notification via e-mail will be deemed given as of the date set forth on the e-mail message.

4.3	In SBC-13STATE Level 3 may designate up to a maximum of ten (10) recipients for Accessible Letter notification via e-mail.

  - In SBC-13STATE, Level 3 shall submit a completed Accessible Letter Recipient Change Request Form to the individual specified on that form to designate in writing each individual’s e-mail address to whom Level 3  requests Accessible Letter notification be sent.  Level 3  shall submit a completed Accessible Letter Recipient Change Request Form to add, remove or change recipient information for any Level 3  recipient of Accessible Letters.  Any completed Accessible Letter Recipient Change Request Form shall be deemed effective ten (10) calendar days following receipt by SBC-13STATE.  SBC-13STATE may, at its discretion, change the process by which the CLEC provides Accessible Letter recipient information.

- Transit Traffic Rate Application

5.1      The Transit Traffic Services rate applies to  all Minutes of Use (“MOUs”) when Level 3 sends Section 251(b)(5) Traffic, ISP-Bound Traffic, IntraLATA Toll  Traffic, CMRS provider-bound traffic and / or 800 IntraLATA Toll Traffic to a  Third Party Terminating Carrier’s network through SBC-12STATE’s tandem switch where an SBC-12STATE End User is neither the originating nor the terminating party.  Level 3 agrees to compensate SBC-12STATE operating as a Transit Service Provider as defined in Section 1.3 at the applicable rates set forth in Appendix Pricing and as specified in Section 5.2 below.

    - Pursuant to the Missouri Public Service Commission Order in Case No. TO-99-483, the Transit Traffic rate element shall not apply to MCA Traffic (i.e., no transiting charges shall be assessed for MCA Traffic) for SBC MISSOURI.

  - Rate Elements  - the following rate elements apply, but the corresponding rates are  specified in Appendix Pricing, attached hereto:

    - Tandem Switching  - Compensation for the use of SBC-12STATE’s Tandem Switch.

    - Tandem Switched Transport - compensation for the transmission facilities between SBC-12STATE’s local tandem and the end offices subtending that tandem.

  - In SBC CONNECTICUT, the Connecticut Transit Traffic Service Rate applies when Level 3   sends Section 251(b)(5) Traffic, ISP-Bound Traffic, IntraLATA Toll Traffic and 800 IntraLATA Toll Traffic to a Third Party Terminating Carrier network through SBC CONNECTICUT’s Tandem.  Level 3   is responsible for payment of the Connecticut Transit Traffic Service Rate.  The Connecticut Transit Traffic Service Rate is only applicable when calls do not terminate to an SBC CONNECTICUT End User.  The Connecticut Transit Traffic Service Rate is specified in Appendix Pricing, attached hereto.

6.0  Transit Traffic Routing

6.1    	Where SBC-12STATE has a Local Tandem Switch separate from an Access Tandem Switch in the local exchange area, Level 3’s originated Section 251(b)(5) Traffic and ISP-Bound Traffic utilizing SBC-12STATE’s Transit Traffic Service will be routed via SBC-12STATE’s Local Tandem Switches, but not at or through any SBC-12STATE Access Tandem Switches.

6.2    Where SBC-12STATE has a Local/IntraLATA Tandem Switch or Local/Access Tandem Switch in the local exchange area, Level 3 originated Section 251(b)(5) Traffic or ISP-Bound Traffic utilizing SBC-12STATE’s Transit Traffic Service will be routed via the appropriate SBC-12STATE Local/IntraLATA Tandem Switch or Local /Access Tandem Switch.

6.3    Where SBC-12STATE has a Local Tandem Switch separate from an Access Tandem Switch in the local exchange area, Level 3 originated IntraLATA Toll Traffic or 800 IntraLATA Toll Traffic utilizing SBC-12STATE’s Transit Traffic Service will be routed via SBC-12STATE’s Access Tandem Switches, but not at or through any SBC-12STATE Local Tandem Switches.

6.4      Where SBC-12STATE has a combined Local/IntraLATA Tandem Switch or Local/Access Tandem Switch in the local exchange area, Level 3 originated IntraLATA Toll Traffic or 800 IntraLATA Toll Traffic utilizing SBC-12STATE’s Transit Traffic Service will be routed via the appropriate SBC-12STATE Local/IntraLATA Tandem Switch or Local/Access Tandem Switch.

6.5     Upon written notification from SBC-13STATE of misrouting of Transit Traffic by Level 3 as identified above, Level 3 will take appropriate action and correct such misrouting within a reasonably practical period of time no longer then 60 days after receipt of notification of such misrouting.

7.0  Direct Trunking Requirements

7.1  	When Transit Traffic from Level 3 through the SBC-12STATE Tandem to another Local Exchange Carrier, CLEC or wireless carrier requires twenty-four (24) or more trunks, upon SBC-12STATE written  request, Level 3  shall establish a direct trunk group or alternate transit arrangement between itself and the other Local Exchange Carrier, CLEC or wireless carrier within sixty (60) calendar days. Level 3 shall route Transit Traffic via SBC-12STATE’s Tandem switches, and not at or through any SBC 12-STATE End Offices.  Once  this trunk group has been established, Level 3 agrees to cease routing Transit Traffic through the SBC-12STATE Tandem to the Third Party Terminating Carrier, unless the parties mutually agree otherwise.