ATTACHMENT 11:  NETWORK INTERCONNECTION ARCHITECTURE

This Attachment 11:  Network Interconnection Architecture to the Agreement describes the technical arrangement by which CLEC and SBC MISSOURI will interconnect their networks in the event that CLEC operates as a Facility-Based Provider in a given SBC MISSOURI  Local Exchange Area.  The arrangements described herein do not apply to the provision and utilization of Lawful UNEs which are addressed in Attachment 6:  Lawful Unbundled Network Elements.

- Definitions:

  - “Access Tandem Switch” is defined as a switching machine within the public switched telecommunications network that is used to connect and switch trunk circuits between and among other central office switches for IXC-carried traffic.

  - “End Office” or “End Office Switch” is a switching machine that directly terminates traffic to and receives traffic from end users purchasing local exchange services.  A PBX is not considered an End Office Switch.

  - “Facility-Based Provider” is defined as a telecommunications carrier that has deployed its own switch and transport facilities.

1.4	“IntraLATA Toll Traffic” is defined as  traffic between one SBC- MISSOURI local calling area  and  another SBC-MISSOURI local calling area or the local calling area of another LEC within one LATA within the respective state.

1.5	“IntraLATA Toll Trunk Group”  is defined as a trunk group carrying IntraLATA Toll Traffic as defined above.

1.6	“ISP-Bound Traffic” is as defined in Attachment 12: Intercarrier Compensation

1.7	“Local Tandem” refers to any Local Only, Local/IntraLATA, or Local/Access Tandem Switch serving a particular LCA (defined below).

1.8	“Local/Access Tandem Switch” is defined as a switching machine within the public switched telecommunications network that is used to connect and switch trunk circuits between and among other central office switches for Section 251(b)(5)/IntraLATA Toll Traffic and IXC-carried traffic.

1.9	A “Local Calling Area” or “LCA” is an SBC MISSOURI local calling area, as defined in SBC MISSOURI’s General Exchange Tariff. LCA is synonymous with “Local Exchange Area” (LEA).

1.10	“Local Interconnection Trunk Groups” are one-way or two-way trunk groups used to carry Section 251(b)(5)/IntraLATA Toll Traffic between CLEC end users and SBC MISSOURI end users.

  - A “Local/IntraLATA Tandem Switch” is defined as a switching machine within the public switched telecommunications network that is used to connect and switch trunk circuits between and among other central office switches for Section 251(b)(5)/IntraLATA Toll Traffic.

1.12	“Local Only Tandem Switch” is defined as a switching machine within the public switched telecommunications network that is used to connect and switch trunk circuits between and among other central office switches for Section 251(b)(5) and ISP Bound Traffic.

1.13	“Offers Service” – At such time as CLEC opens an NPA/NXX, ports a number to serve an end user, or pools a block of numbers to serve end users.

1.14	“Remote End Office Switch” is an SBC MISSOURI switch that directly terminates traffic to and receives traffic from end users of local Exchange Services, but does not have full feature, function and capability of an SBC MISSOURI End Office Switch.  Such features, functions, and capabilities are provided to the SBC MISSOURI Remote End Office Switch from an umbilical to the SBC MISSOURI Host End Office.

1.15	Section 251(b)(5) Traffic is as defined in Attachment 12:  Intercarrier Compensation.

1.16	“Section 251(b)(5)/IntraLATA Toll Traffic” shall mean for purposes of this Attachment, (i) Section 251(b)(5) Traffic, (ii) ISP-Bound Traffic, (iii) IntraLATA Toll Traffic originating from an end user obtaining local dialtone from CLEC where CLEC is both the Section 251(b)(5) Traffic and intraLATA toll provider, and/or (iv) IntraLATA Toll Traffic originating from an end user obtaining local dialtone from SBC-MISSOURI where SBC-MISSOURI is both the Section 251(b)(5) Traffic and intraLATA toll provider.

1.17	A “Tandem Serving Area” or “TSA” is  an SBC MISOURI area defined by the sum of all local calling areas served by SBC MISSOURI End Offices that subtend an SBC MISOURI tandem for Section 251(b)(5)/IntraLATA Toll Traffic as defined in the LERG.

- Requirements for Establishing Points of Interconnection.

Section 2.1 through Section 2.4 are the Parties’ requirements for establishing a Point of Interconnection (POI) for the Exchange of Section 251(b)(5)/IntraLATA Toll Traffic.

2.1 	The Parties will interconnect their network facilities, using any technically feasible method, at a minimum of one CLEC designated Point of Interconnection (POI) within the LATA where CLEC Offers Service.  The POI(s) shall be established at any technically feasible point within SBC MISSOURI’s network (including, without limitation, tandems, end offices, outside plant, customer premises, and at “carrier hotels”).

2.1.1	A “Single POI” is a single point of interconnection within a LATA on SBC MISSOURI’s  network that is established to interconnect SBC MISSOURI’s network and CLEC’s network for the exchange of Section 251(b)(5)/IntraLATA Toll Traffic.

2.1.2	The Parties agree that CLEC has the right to choose a single POI or multiple POIs.

2.1.3	When CLEC has established a single POI (or multiple POIs) in a LATA,  CLEC agrees to establish an additional POI:

(i)  in any SBC MISSOURI  TSA separate from any existing POI arrangement when traffic to/from that  SBC MISSOURI TSA exceeds twenty-four (24) DS1s at peak over three (3) consecutive months, or

(ii) at an SBC MISSOURI End Office in a local calling area not served by an SBC MISSOURI tandem for Section 251(b)(5)/IntraLATA Toll Traffic when traffic to/from that local calling area exceeds twenty-four (24) DS1s at peak over three (3) consecutive months

2.1.4	The additional POI(s) will be established within 90 days of notification that the threshold has been met.

2.1.5	 The following are the Parties’ responsibilities when new POIs need to be established:

(i)	The new POI location shall be at CLEC’s designation.

(ii)	Additional POIs shall be established, using any technically feasible method, at any technically feasible point within SBC MISSOURI’s network (including, without limitation, tandems, end offices, outside plant, customer premises, and at “carrier hotels”).

(iii)	Each party will be responsible for providing the necessary equipment and facilities on their side of the POI.

(iv)	The POI(s) will be identified by street address and Vertical and Horizontal (V and H) Coordinates.

- Trunking Requirements per LCA

  - At such time as CLEC Offers Service for the exchange of Section 251(b)(5)/IntraLATA Toll Traffic in an LCA, CLEC shall establish the necessary Local Interconnection Trunk Groups (in accordance with Appendix ITR) to:

    - Each SBC MISSOURI Local Tandem in the LCA where CLEC Offers Service when there are SBC MISSOURI Local Tandem(s) in the LCA where CLEC Offers Service. DEOT group(s) shall be provisioned as one-way or two-way trunks and used as two-way trunks.

  - When CLEC Offers Service in an LCA that has at least one SBC MISSOURI Local Tandem, and the Section 251(b)(5)/IntraLATA Toll Traffic between CLEC and an SBC MISSOURI End Office which subtends an SBC MISSOURI Local Tandem in the LCA exceeds 24 DS0s at peak over a period of three consecutive months , CLEC shall may, at its election, establish a Direct End Office Trunk Group (Local Interconnection Trunk Group that terminates to a SBC MISSOURI End Office also known as a “DEOT” group) to that SBC MISSOURI End Office. SBC MISSOURI will not charge CLEC for the transport facility(ies) associated with the DEOT from the CLEC designated POI to the SBC MISSOURI End Office(s) where the CLEC DEOT was established.

  - When the LCA in which CLEC Offers Service for the exchange of Section 251(b)(5)/IntraLATA Toll Traffic is served only by an SBC MISSOURI Remote End Office Switch, CLEC shall DEOT to the appropriate SBC MISSOURI Host End Office Switch.

3.4	DEOT group(s) shall be provisioned as one-way or two-way trunks and used as two-way trunks.

- Provision of Information

  - In order to establish or designate any POI and associated trunks and transport facilities under this Agreement, CLEC shall provide all applicable network information on forms acceptable to SBC MISSOURI (as set forth in SBC MISSOURI’s CLEC Handbook, published on the CLEC website.)

- ASR Control for two-way trunk groups

  - CLEC shall have administrative and order control (e.g., determination of trunk group size) of all two-way trunk groups provisioned between CLEC and SBC MISSOURI with the limited exception detailed in section 5.3 below.

  - This only applies to the extent that it does not require SBC MISSOURI to redesign its network configuration.

  - SBC MISSOURI reserves the right to issue an ASR on CLEC’s behalf in the event CLEC is non-responsive to a TGSR for underutilized trunk groups as outlined in Appendix ITR.  At no other time shall SBC MISSOURI be allowed to issue ASRs on CLEC’s behalf.

- Ancillary Services

  - Where CLEC requires ancillary services (e.g., Directory Assistance, Operator Services, E911), additional POIs may be required for interconnection to such ancillary services.

  - CLEC is solely responsible for the facilities that carry OS/DA, 911, mass calling and Meet-Point trunk groups.  The trunking requirements for these are specified in Appendix ITR.

- Signaling

  - Trunks will utilize Signaling System 7 (SS7) protocol signaling when such capabilities exist within the SBC MISSOURI network.

  - Multifrequency (MF) signaling will be utilized in cases where SBC MISSOURI switching platforms do not support SS7.

- Interconnection Methods

  - Where CLEC seeks to interconnect with SBC MISSOURI for the purpose of mutually exchanging Section 251(b)(5)/IntraLATA Toll Traffic between networks, CLEC may use any of the following methods of obtaining interconnection detailed in Appendix Network Interconnection Methods (NIM) attached hereto and incorporated herein.  Such methods include but are not limited to:

    - Physical Collocation

    - Virtual Collocation

    - SONET Based

    - Fiber Meet Point

    - Leasing of facilities from a third party

    - CLEC self-buildout

    - Any other mutually agreeable methods of obtaining interconnection.

9.0	In addition, the Parties agree to the interconnection and trunking requirements listed in Appendix Interconnection Trunking Requirements (ITR), which is attached hereto and made a part hereof.