INDEX

Sheet No.

1	Index

2	Map of Service Area

3	Legal Description of Service Area

4	Schedule of Rates

5	Schedule of Service Charges

Rule No.	Rule Title

6	1	Definitions

9	2	General Rules and Regulations

10	3	Company Employees and Customer Relations

11	4	Applications for Service

12	5	Inside Piping and Water Service Lines

15	6	Improper or Excessive Use

16	7	Discontinuance of Service by Company

21	8	Termination of Water Service at Customer's Request

22	9	Interruptions in Service

23	10	Bills for Service

25	11	Meters and Meter Installations

28	12	Meter Tests and Test Fees

29	13	Bill Adjustments Based on Meter Tests

30	14	Extension of Water Mains

33	15	Deposits

Map of Service Area

Legal Description of Service Area

A fractional part of the South 1/2 of the NW 1/4 of Section 20, Township 38 North, Range 7 West more particularly described as follows: Beginning at the Northwest corner of the South 1/2 of the NW l/4 of Section 20, Township 38 North, Range 7 West, Thence South 88 degrees 32 minutes East 30 feet to the point of beginning of the tract herein described; thence South 88 degrees 32 minutes East along the North line of the South 1/2 of the NW l/4 of Section 20, Township 38 North, Range 7 West 2,644.93 feet to the Northeast corner of the South 1/2 of the NW 1/4 of Section 20, Township 38 North, Range 7 West; thence South 0 degrees 03 minutes West along the East line of the South 1/2 of the NW 1/4 of Section 20, Township 38 North, Range 7 West 1,006.32 feet; thence North 88 degrees 32 minutes West 2,644.93 feet to the East ROW of Phelps County Highway "V"; thence North 0 degrees 03 minutes East along the East ROW of Phelps County Highway "V" 1,006.32 feet to the point of beginning; all in Phelps County, Missouri.

Adding

A fractional part of the South 1/2 of the NW 1/4 of Section 20, Township 38 North, Range 7 West, and a fractional part of the South 1/2 of the NE 1/4 of Section 19, Township 38 North, Range 7 West,  more particularly described as follows: Beginning at the Northwest corner of the South 1/2 of the NW l/4 of Section 20, Township 38 North, Range 7 West, Thence South 88 degrees 32 minutes East 30 feet, thence  South 0 degrees 03 minutes West along the East ROW of Phelps County Highway "V" 1,006.32 feet to the point of beginning of the tract herein described; thence North 88 degrees 32 minutes West 700 feet to a point that is in Section 19, Township 38 North, Range 7 West; thence North 0 degrees 03 minutes East 345 feet to a point; thence South 88 degrees 32 minutes East 700 feet to a point on the East ROW of Phelps County Highway "V"; thence South 0 degrees 03 minutes West along the East ROW of Phelps County Highway "V" 345 feet to the point of beginning; all in Phelps County, Missouri.

The entire service area contains approximately 66.65 acres.

Schedule of Rates

Availability:

Available to any Customer located in the customer’s service area

Rates:

Minimum Monthly Charge (First 1,000 gallons or less) 			$17.25

Water Usage Charge (All over 1,000 gallons)					$3.77 per 1,000 gallons

Taxes:

Any applicable Federal, State or local taxes computed on billing basis shall be added as separate

items in rendering each bill.

Schedule of Service Charges

Discontinuance of Service for Non-Payment of Bill

For Customers with Gladlo water service:

Disconnection / turn-off charge 					$15.00

Reconnection / turn-on charge 					$15.00

For Customers without Gladlo water service:

Reconnection charges for discontinued sewer service for non-payment of bill shall be the actual

expenses incurred by the Company for disconnection and reconnection.

New Service Connection

Tap-on 										Actual Cost

Inspection of service connection 					$25.00

Late Charge

For any bill not paid within the period stated thereon, a late charge in the amount of three dollars

($3.00) per month, not to exceed three months or nine dollars ($9.00) per Customer, shall be

added to the total amount due plus all disconnection and reconnect costs, collection costs, and

reasonable fees for collections.

Returned Check Charge

A charge of twenty-five dollars ($25.00) will be added to any account for a check returned from the bank unpaid.

Rule 1  DEFINITIONS

- An "APPLICANT" is a person, firm, corporation, governmental body, or other entity which has applied for service; two or more APPLICANTS may make one application for a water main extension and be considered one APPLICANT.

- The "COMPANY" is Gladlo Water & Sewer Company, Inc., acting through its officers, managers, or other duly authorized employees or agents.

- The “CURB STOP” is a valve on the Service Connection, located at or near the Customer’s property line, and used to shut off water service to the premises.  The Curb Stop is owned and maintained by the Company.

- A "CUSTOMER" is any person, firm, corporation or governmental body which has contracted with the Company for water service or is receiving service from Company, or whose facilities are connected for utilizing such service, and except for a guarantor is responsible for payment for service.

- The "DATE OF CONNECTION" shall be the date of the permit for installation and connection issued by the Company.  In the event no permit is taken and a connection is made, the date of connection shall be based on available information such as construction/occupancy permits, electric service turn-on date, or may be the date of commencement of construction of the building upon the property.

- A "DEVELOPER" is any person, firm, corporation, partnership or any entity that, directly or indirectly, holds title to, or sells or leases, or offers to sell or lease, or advertises for sale or lease, any lots in a subdivision.

- "DISCONTINUANCE OF SERVICE" is the intentional cessation of service by the Company not requested by the Customer.

- The "MAIN" is a pipeline which is owned and maintained by the Company, located on public property or private easements, and used to transport water throughout the Company's service area.

- The "METER" is a device, owned by the Company, used to measure and record the quantity of water that flows through the service line, and is installed in the outdoor meter setting, or inside the Customer’s building where the water service line enters through a foundation wall.

- The "METER SETTING" is a place either in the service connection or building plumbing for a water meter to be installed.  An outdoor meter setting is located at or near the property line, and includes the meter box, meter yoke, lid, and appurtenances, all of which shall be owned and maintained by the Company.  Indoor meter settings are located inside the Customer’s premises where the water service line enters the foundation wall either installed directly in the piping or in a meter yoke.

- A “RETURNED CHECK” is a check that is returned to the Company from any bank unpaid for any reason.

- A “SEASONAL CUSTOMER” is a Customer who is absent from the premises and may turn off, or request the Company turn off, water service temporarily.  All Rates, Rules and Regulations within this tariff continue to apply to “Seasonal Customers” during periods of seasonal absence or turn-off.

- The "SERVICE CONNECTION" is the pipeline connecting the main to the Customer's water service line and includes the curb stop, or outdoor meter setting and all necessary appurtenances located at or near the property line, or at the property line if there is no curb stop or outdoor meter setting.  If the property line is in a street, and if the curb stop or meter setting is not located near the edge of the street abutting the Customer’s property, the service connection shall be deemed to end at the edge of the street abutting the Customer's property.  The service connection shall be owned and maintained by the Company.

- A "SUBDIVISION" is any land in the state of Missouri which is divided or proposed to be divided into two or more lots or other divisions of land, whether contiguous or not, or uniform in size or not, for the purpose of sale or lease, and includes resubdivision thereof.

- "TERMINATION OF SERVICE" is cessation of service requested by the Customer.

- “TURN-OFF” is the act of turning water service off by physically turning a valve such that water is unavailable to a Customer’s premises.

- “TURN-ON” is the act of turning water service on by physically turning a valve to allow water to be available to a Customer’s premises.

- The word "UNIT” or “LIVING UNIT” shall be used herein to define the premises or property of a single water consumer, whether or not that consumer is the Customer.  It shall pertain to any building whether multi-tenant or single occupancy, residential or commercial, or owned or leased.  Each mobile home in

a mobile home park and each rental unit of a multi-tenant rental property are considered as separate units for each single family or firm occupying same as a residence or place of business.

- The "WATER SERVICE LINE" is a pipe with appurtenances installed, owned and maintained by the Customer, used to conduct water to the Customer's unit from the property line, curb stop or outdoor meter setting, including the connection to the curb stop or meter setting.  If the property line is in a street, then the water service line shall be deemed to begin at the edge of the street abutting the Customer's property.
Rule 2  GENERAL RULES & REGULATIONS

- Every applicant, upon signing an application for any water service rendered by the Company, or any Customer upon taking of water service, shall be considered to have expressed consent to be bound by these Rates, Rules and Regulations.

- The Company's Rules and Regulations governing rendering of service are set forth in these numbered sheets.  The rates applicable to appropriate water service or service in particular service areas are set forth in rate schedules and constitute a part of these Rules and Regulations.

- The Company reserves the right, subject to authority of the Missouri Public Service Commission, to prescribe additional Rates, Rules or Regulations or to alter existing Rates, Rules or Regulations as it may from time to time deem necessary and proper.

- After the effective date of these Rules and Regulations, all new facilities, construction contracts, and written agreements shall conform to these Rules and Regulations, and in accordance with the statutes of the state of Missouri and the Rules and Regulations of the Missouri Public Service Commission.  Pre-existing facilities that do not comply with applicable Rules and Regulations may remain, provided that their existence does not constitute a service problem or improper use, and reconstruction is not practical.

- The point of delivery of water service shall be at the connection of the Customer’s service line to the Company’s service connection.

- The Company shall have the right to enter upon the Customer's premises for the purpose of inspecting for compliance with these Rules and Regulations.  Company personnel shall identify themselves and such inspections shall be conducted during reasonable hours.

Rule 3  COMPANY EMPLOYEES AND CUSTOMER RELATIONS

- Employees or agents of the Company are expressly forbidden to demand or accept any compensation for any services rendered to its Customers except as covered in the Company's Rules and Regulations.

- No employee or agent of the Company shall have the right or authority to bind it by any promise, agreement or representation contrary to the intent of these Rules and Regulations.

- The Company shall not be responsible for damages caused by any failure to maintain water pressure or water quality, or for interruption, if such failure or interruption is without willful default or negligence on its part.

- The Company shall not be liable for damages due to, or interruptions caused by, defective piping, fittings, fixtures and appliances on the Customer’s premises and not owned by the Company.

- The Company shall not be liable for damages due to Acts of God, civil disturbances, war, government actions, or other uncontrollable occurrences.

Rule 4  APPLICATIONS FOR SERVICE

- A written application for service, signed by the Customer, stating the type of service required and accompanied by any other pertinent information, will be required from each Customer before service is provided to any unit.

- If service is requested at a point not already served by a main of adequate capacity, a main of adequate size shall be extended as may be necessary in accordance with Rule 14.

- When, in order to provide the service requested a main extension or other construction or equipment expense is required, the Company may require a written contract.  Said contract may include, but not be limited to, the obligations upon the Company and the applicant, and shall specify a reasonable period of time necessary to provide such service.

Rule 5  INSIDE PIPING AND WATER SERVICE LINES

- The Company will provide water service at the outdoor meter, at the curb stop if an indoor meter setting is utilized; or at the property line if neither an outdoor meter nor a curb stop exists at or near the property line, or at the edge of the street if such property line is in the street.  Separate buildings shall be served through separate water service lines if they are not on one lot that cannot be subdivided.

- The service connection from the water main to the Customer's property line shall be owned and maintained by the Company.  Construction of the service connection, outdoor meter setting and curb stop shall be accomplished in one of the following ways at the Customer’s option:

  - The Company will construct the service connection, outdoor meter setting and curb stop, as necessary, and make the connection to the main, within three (3) business days of an application for service, or within the time period specified in an application for service (See Rule 4).  The Customer shall be responsible for payment of the New Service Connection Fee, as specified by or provided for in the Schedule of Service Charges; or,

  - The Customer may install, or have installed by a professional contractor or plumber, the service connection from the water main to the meter setting, and make the connection to the main, subject to prior approval of the Company; or,

  - The Customer may install, or have installed by a professional contractor or plumber, the service connection from the water main to the meter setting, and the Company will tap the main and connect the service connection.  The Customer shall be responsible for payment of a New Service Connection Fee as specified by or provided for in the Schedule of Service Charges.

- A service connection installation constructed by the Customer as provided for in paragraphs B. 2. or 5 B. 3., above, is subject to inspection by the Company.  The Service Connection Inspection Fee as specified in the Schedule of Service Charges shall apply if the Company must make a trip solely to conduct an inspection of a service connection constructed by the Customer, and shall not apply if the inspection of a service connection is accomplished at the same time as a tap is made for the Customer, or the same time as an inspection of the water service line as provided for in paragraph D., below, or if the Company installs the service connection as provided in 5 B. 1., above.

- Water service line construction and maintenance from the property line, curb stop or meter setting, including the connection to the curb stop or meter setting, to the building shall be the responsibility of the Customer, and is subject to inspection by the Company.  The Customer shall be responsible for any applicable fees as listed in the Schedule of Service Charges.  Customers shall be responsible for the cost of repairing any damage to the Company's mains, curb stops, valve boxes, meters, and meter installations caused by the Customer, Customer’s agent, or tenant.

- Existing water service lines and service connections may be used in connecting with new buildings only when they are found by examination and testing not to constitute a hazard to the health and safety of any Customer or the Company's facilities.

- The water service line shall be brought to the unit at a depth of not less than thirty-six inches (36”) and have a minimum inside diameter of three-quarters inch (3/4”).  The Customer is responsible for the determination of whether or not a larger size is needed to provide adequate flow to the unit.  A valve must be installed in the service line where it enters the unit.  This valve must be kept in good repair in order to shut off the water supply and drain the inside plumbing, if necessary.

- Water service lines and inside piping shall be of material conforming to recognized standards for potable water service and shall have a pressure rating of at least one hundred sixty (160) psi working pressure.

- The Company will not install a service connection to a vacant lot if such lot is not intended and recognized by the Company to be for intermittent use such as camping or picnic activity in a recreational subdivision, and the Customer installs a frost-free lockable hydrant at any point of use.

- Any change in the location of an existing service connection requested by the Customer shall be made by the Company or with the Company’s approval, at the Customer’s expense.

- The Company shall have the right to enter the Customer's premises, after reasonable notice, for the purposes of inspection to ensure compliance with these Rules and Regulations.  Company personnel shall identify themselves and make these inspections only at reasonable hours.

- Neither water service lines nor service connections may be extended along public streets or roadways or through property of others in connecting with the Company's mains.  The service connection may, however, extend through the water main easement and roadway easement as necessary in order to be connected to a main located across and adjacent to a street in front of the Customer's living unit.  The service connection and service line must be laid in a straight line and at right angles to the main and the face of the structure or as nearly so as possible.  Any deviation from this because of physical obstruction, landlocked property, or a clear impossibility to construction a future main extension for further subdivision development or additional future customers, will be at the discretion of the Company.

- Any Customer having a plumbing arrangement, or a water-using device that could allow backsiphonage of any chemical, petroleum, process water, water from a questionable supply, or other substance that could create a health hazard or damage to the water system; or, any Customer's plumbing classified as an actual or potential backflow hazard in the Regulations of the Missouri Department of Natural Resources shall be required to install and maintain a backflow prevention device.  This rule may also apply to Customers on whose premises it is impossible or impractical for the Company to perform a cross connection survey.  The device, installation, location and maintenance program shall be approved by the Company.

Rule 6  IMPROPER OR EXCESSIVE USE

- No Customer shall be wasteful of the water supplied to the unit by the Customer’s willful action or inaction.  It shall be the responsibility and duty of each Customer to maintain all piping and fixtures at the unit in a good and efficient state of repair at all times.

- No Customer shall make or cause to be made a cross connection between the potable water supply and any source of chemical or bacterial contamination or any other water supply.  The Company shall deny or discontinue service where Customer's water service line or inside piping may, in the opinion of the Company, cause a cross-connection with non-potable water or otherwise jeopardize the health and safety of other Customer’s or the Company's facilities.

- The Customer shall not make or cause to be made a connection to a device that will result in excessive water demand or excessive shock, such as water-hammer, to the Company's mains.

- The Customer shall not tamper with, remove, or willfully damage a water meter or attempt to operate the shutoff cock on the service connection or meter yoke, or allow any such action.  Licensed plumbers may operate such valves in order to work on the Customer’s premises and to test their work, but must leave such valves open or closed as found.

- The Customer shall not attempt to take unmetered water from the Company mains either by an unauthorized tap or direct connection to service connection nor by connection to a fire hydrant.

- Customers will not be permitted to supply water in any way to premises other than the service address, nor to permit others to use their hose or attachments, nor leave them exposed to use by others without permission from the Company.

Rule 7  DISCONTINUANCE OF SERVICE BY COMPANY

- The Company may discontinue service for any of the following reasons:

  - Non-payment of a delinquent account not in dispute; or

  - Failure to post a security deposit or guarantee acceptable to the utility; or

  - Unauthorized interference, diversion or use of the utility service situated or delivered on or about the Customer's premises; or

  - Misrepresentation of identity in obtaining utility service; or

  - Enclosing or obstructing any meter so as to make reading or repairs unreasonably difficult, or

  - Failure to comply with the terms and conditions of a settlement agreement; or

  - Refusal after reasonable notice to grant access at reasonable times to equipment installed upon the premises of the Customer for the purpose of inspection, meter reading, maintenance or replacement; or

  - Violation of any of these Rules and Regulations on file with and approved by the Missouri Public Service Commission, or for any condition which adversely affects the safety of the Customer or other persons, or the integrity of the utility's delivery system; or

  - Non-payment of a sewer bill issued by the Company or by a sewer utility requesting discontinuance of water service by an agreement between the Company and such sewer utility.  When water service is discontinued for non-payment of a sewer bill and if the sewer bill is not issued by the Company, any service charges for turn on/off or disconnection/reconnection within these Rules and Regulations shall not apply, and notice to the Customer shall be provided by rules and procedure applicable to the Customer's sewer service in lieu of notification required by these Rules and Regulations.

- None of the following shall constitute sufficient cause for the Company to discontinue service:

  - The failure of the Customer to pay for merchandise, appliances, or service not subject to Commission jurisdiction as an integral part of the utility service provided by the Company; or

  - The failure of the Customer to pay for service received at a separate metering point, residence, or location.  In the event of discontinuance or termination of service at a separate residential metering point, residence, or location in accordance with these Rules and Regulations, the Company may transfer and bill any unpaid balance to any other residential service account of the Customer and may discontinue service after twenty-one (21) days after rendition of the combined bill, for nonpayment, in accordance with this rule; or

  - The failure of the customer to pay for a different class of service received at the same or different location.  The placing of more than one (1) meter at the same location for the purpose of billing the usage of specific devices under operational rate schedules or provisions is not construed as a different class of service for the purpose of this rule; or

  - The failure to pay the bill of another customer, unless the customer whose service is sought to be discontinued received substantial benefit and use of the service billed to the other customer; or

  - The failure of a previous owner or occupant of the premises to pay an unpaid or delinquent bill except where the previous occupant remains an occupant of the living unit; or

  - The failure to pay a bill correcting a previous underbilling, whenever the customer claims an inability to pay the corrected amount, unless a utility has offered the customer a payment arrangement equal to the period of underbilling.

- The Company may discontinue service after notice by first class mail is sent to the Customer at least ten (10) days prior to the date of the proposed discontinuance.  Service of notice by mail is complete upon mailing.  If written notice is hand delivered to the Customer, it shall be done at least ninety-six (96) hours prior to discontinuance.  If the Company intends to discontinue service to a multi-tenant dwelling with occupants who are not customers, a notice shall also be conspicuously posted in the building ten (10) days prior to the proposed discontinuance, along with information pertaining to how one or more of the tenants may apply to become customers.  Discontinuance shall occur within thirty (30)  calendar days after the date given as the discontinuance date, shall occur between the hours of 8:00 a.m. and 4:00 p.m., and shall not occur on a day when the Company will not be available to reconnect service or on a day immediately preceding such a day.

- A discontinuance notice provided to a customer shall include:

  - The name and address of the Customer, the service address if different than the Customer’s address; and

  - A statement of the reason for the proposed discontinuance of service and the cost for reconnection; and

  - How the customer may avoid the discontinuance; and

  - The possibility of a payment agreement it the claim is for a charge not in dispute and the Customer is unable to pay the charge in full at one time; and

  - A telephone number the Customer may call from the service location without incurring toll charges and the address and any available electronic contact information of the utility prominently displayed where the customer may make an inquiry.

- The Company shall make reasonable efforts to contact the Customer, at least twenty-four (24) hours prior to any discontinuance, regarding the reason(s) for discontinuance of service, and the resolution.  If discontinuance of service would affect an occupant who is not the Company's Customer, or is not responsible for payment of the bill, then the Company shall make reasonable efforts to inform such occupant(s).

- The Company shall postpone the discontinuance if personnel will not be available to restore service the same day, or if personnel will not be available to restore service the following day.  The Company also shall postpone discontinuance if a medical emergency exists on the premises, however the postponement may be limited to twenty-one (21) days, and the Company may require proof of a medical emergency.

- The Company shall have the right to enter the Customer's premises for purposes of discontinuance of service in compliance with these Rules and Regulations.  Discontinuance of service will be made during reasonable hours.  Company personnel shall identify themselves and announce the intention to discontinue service, or leave a conspicuous notice of the discontinuance.  The Company shall have the right to communicate with the owner of the Customer’s Unit for purposes of gaining access to the property for discontinuance of service in accordance with the Missouri Public Service Commission’s billing practices, but any extra costs for arranging such access shall not be charged to the Customer’s account.

- The provisions of paragraphs C. and E. above may be waived if safety of

Company personnel while at the premises is a consideration.

- Discontinuance of service to a unit for any reason shall not prevent the Company from pursuing any lawful remedy by action at law or otherwise for the collection of monies due from the Customer.

- In case the Company discontinues its service for any violation of these Rules and Regulations, then any monies due the Company shall become immediately due and payable.

- The Company has the right to refuse or to discontinue service to any unit to protect itself against fraud or abuse.

- The Company shall deal with Customers, handle Customer accounts, and manage discontinuance of service procedures in accordance with the Missouri Public Service Commission's Utility Billing Practices.

- Applicable Turn-off and turn-on charges are specified in the Schedule of Service Charges.

Rule 8  TERMINATION OF WATER SERVICE AT CUSTOMER'S REQUEST

- Service will be terminated at the Customer's request, by giving not less than twenty-four (24) hours notice to the Company during its regular office hours.  The Company shall, on the requested day, read the Customer's meter and charges for water service rendered up to and including the time of termination shall be computed and will become due and payable immediately.

- A Customer may request temporary turn-off by the Company for the Customer’s own convenience; however, the Customer shall still be charged for service at the appropriate rate as specified in the Schedule of Rates during the time the service is turned off.

- Turn-off and turn-on charges shall apply, and are specified in the Schedule of Service Charges.

- A Customer who requests termination of service, but returns to the premises and requests water service within nine (9) months of such termination, at the Company’s discretion may be deemed to have been a seasonal customer, and applicable charges incurred during the period of absence may apply.

Rule 9  INTERRUPTIONS IN SERVICE

- The Company reserves the right to discontinue water in its mains at any time, without notice, for making emergency repairs to the water system.  Whenever service is interrupted for scheduled repairs or maintenance, Customers affected by such interruptions will be notified in advance whenever it is possible to do so.  Every effort will be made to minimize interruption of service.

- No refunds of charges for water service will be made for interruptions of service unless due to willful misconduct of the Company.

- In order to avoid service problems when extraordinary conditions exist, the Company reserves the right, at all times, to determine the limit of and regulate water usage in a reasonable and non-discriminatory manner.

Rule 10  BILLS FOR SERVICE

- The charges for water service shall be at the rates specified in the Schedule of Rates in these Rules and Regulations.  Other applicable service charges are set forth in the Schedule of Service Charges in these Rules and Regulations.

- A Customer who has made application for, or is receiving the benefit of, water service to a unit shall be responsible for payment for all water service provided to the Customer at said unit from the date of connection until the date requested by the Customer by proper notification to the Company to terminate service.

- Each Customer is responsible for furnishing the Company with the correct address. Failure to receive bills will not be considered an excuse for non-payment nor reason to permit an extension of the date when the account would be considered delinquent.  Bills and notices relating to the Company or its business will be mailed or delivered to the mailing address entered in the Customer's application unless the Company is notified in writing by the Customer of a change of address.

- Payments shall be made at the office of the Company or at such other places conveniently located as may be designated by the Company, by ordinary mail, or by electronic methods employed by the Company.  Payment must be received by the close of business on the date due, unless the date due falls on a non-business day in which case payment must be received by the next business day.

- Neither the Company nor the Customer will be bound by bills rendered under mistake of fact as to the quantity of service rendered or as a result of clerical error. Customers will be held responsible for charges based on service provided.

- A separate bill shall be rendered for each Customer with itemization of all water service charges.  All bills for service shall state the due date.  The Company shall render bills monthly.

- Monthly bills shall be due twenty-one (21) calendar days from the date of rendition, unless such due date falls on a Sunday, a legal holiday, or other day when the office is closed, in which case the due date shall be extended to the next business day.  Bills unpaid after the stated due date will be delinquent and the Company shall have the right to discontinue service in accordance with Rule 7.    Delinquent bills may be subject to a late charge as provided in the Schedule of Service Charges.  The Company shall not be required to restore or connect any new service for such delinquent Customers until the unpaid account due the Company under these Rules and Regulations has been paid in full or arrangements satisfactory to the Company have been made to pay said account.

- When bills are rendered for a period of less than a complete billing period due to the connection or termination of service, the billing shall be the monthly or quarterly minimum plus an amount based on the water used at the commodity (water usage) rate or one-half (1/2) of the flat rate if applicable.

- All billing matters shall be handled in accordance with the Missouri Public Service Commission's Rules and Regulations regarding Utility Billing Practices, 4 CSR 240-13.

Rule 11  METERS AND METER INSTALLATIONS

- When water meters are utilized for billing, the Company shall furnish and install a suitable meter for each Customer, and the Company's installed meter shall be the standard for measuring water used to determine the bill.  All meters shall be furnished, installed, maintained and removed by the Company and shall remain its property.

- The Company shall have the right to determine, on the basis of the Customer's flow requirements, the type and size of meter to be installed and location of same.  No meter size selection will be based solely on the size of the Customer’s service line.  If flow requirements increase or decrease subsequent to installation and a larger or smaller meter is requested by the Customer, the cost of installing such larger or smaller meter shall be paid by the Customer.

- Domestic water service to any one Customer at a single premises shall be furnished through a single service connection.  Individual units of a multi-unit building may have separate connections and meter installations only if each unit has separate plumbing, ground-level space, an individual service connection and meter installation location, and frontage to a Company-owned main.  For multi-unit buildings with one service connection and meter installation, the inside piping may be rearranged at the Customer's own expense so as to separate the units and meter tenants, then divide the bill accordingly.

- The owners of premises wherein meters are located shall be held responsible for the safekeeping of the Company’s meters and metering appurtenances, and are required to keep meters located within their property accessible to the Company

for reading and for meter changeouts.  If a Customer limits accessibility, or fails to protect a meter against damage, the Company may discontinue service and/or refuse to supply water until accessibility is restored and the Company is paid for any such damage.  The amount of the charge shall be the cost of the necessary replacement parts and the labor cost necessary to make the repair.

- If the Company determines that no suitable outdoor location is available, then the meter may be installed inside the Customer’s premises where the water service line enters the building and just downstream of the inside shutoff valve. The Company shall install a curb stop within the service connection at or near the property line as practical.  When the meter is installed inside the Customer’s premises, the Customer will either provide a meter yoke to accept installation of the Company’s meter, or provide proper fittings for the house plumbing pipe to allow for direct installation of the Company’s meter, along with a proper grounding strap installed around the meter to prevent electric charge build-up on either side of the meter or while a meter is removed.  If installation in a special setting is necessary, the excess cost of installation shall be paid by the Customer.

- If an existing basement meter location is determined by the Company to be inadequate or inaccessible, then the Customer must provide for the installation of a meter to be located at or near the Customer's property line.  The Customer shall furnish or obtain from the Company, as appropriate, the necessary meter installation appurtenances conforming to the Company's specifications, and the cost of said appurtenances and labor shall be paid by the Customer.

- Approved meter installation locations in dry basements, sufficiently heated to keep the meter from freezing, may remain provided the meter is readily accessible, at the Company's and Customer's convenience as determined by the Company, for servicing and reading and the meter space provided is located where the service line enters the building.  The Company may, at its discretion, require the Customer to install a remote reading device at an approved location, for the purpose of reading the meter.  It is the responsibility of the Customer and/or the owner of the premises to provide a location for the water meter which, in the event of water discharge as a result of leakage from the meter or couplings, will not result in damage.  The Company's liability for damages to any and all property caused by such leakage shall in no event exceed the price of water service to the affected premises for one average billing period in the preceding year.  Where damage is caused by the negligence of Company personnel at the premises, this limitation will not apply.  If a Customer refuses to provide an accessible location for a meter as determined by the Company, the Company will notify the Staff of the Water and Sewer Unit of the Missouri Public Service Commission before ultimately refusing service or proceeding to discontinue service.

- The Customer shall promptly notify the Company of any defect in, or damage to, the meter setting.

- Any change in the location of any existing meter or meter setting at the request of the Customer shall be made at the expense of the Customer, and with the approval of the Company.

Rule 12  METER TESTS AND TEST FEES

- Any Customer may request the Company to make a special test of the accuracy of the meter through which water is supplied to the Customer.  This test will be made in accordance with water industry test procedures, and to check for accuracy as required by Regulations of the Missouri Public Service Commission.

- The Company reserves the right to remove and test a meter at any time and to substitute another in its place.  In case of a dispute involving a question as to the accuracy of the meter, a test will be made by the Company upon the request of the Customer without charge if the meter has not been tested within twelve (12) months preceding the requested test; otherwise, an approved charge will be made if the test indicates meter accuracy within five percent (5%).

- A meter test requested by the Customer may be witnessed by the Customer or the Customer’s duly authorized representative, except for tests of meters larger than two inch (2”) inlet, which will be conducted by the water manufacturer.  A certified copy of the test report will be provided to the Customer.

- If a test shows an average error of more than five percent (5%), billings shall be adjusted in accordance with Rule 13.

Rule 13  BILL ADJUSTMENTS BASED ON METER TESTS

- Whenever any test by the Company of a meter while in service or upon its removal from service shall show such meter to have an average error of more than five percent (5%) on the test streams prescribed by the Missouri Public Service Commission, the Company shall adjust the Customer's bills by the amount of the actual average error of the meter and not the difference between the allowable error and the error as found.  The period of adjustment on account of the under-registration or over-registration shall be determined as follows:

  - Where the period of error can be shown, the adjustment shall be made for such period; or

  - Where the period of error cannot be shown, the error found shall be considered to have existed for three (3) months preceding the test.

- If the meter is found on any such test to under-register, the Company may render a bill to the Customer for the estimated consumption not covered by bills previously rendered during the period of inaccuracy as above outlined.  Such action shall be taken only when the Company was not at fault for allowing the inaccurate meter to remain in service.

- If the meter is found on any such test to over-register, the Company shall refund to the Customer any overcharge caused during the period of inaccuracy as above defined.  The refund shall be paid within a reasonable time and may be in the form of a bill credit.

Rule 14  EXTENSION OF WATER MAINS

- This rule shall govern the extension of mains by the Company within its certified area where there are no water mains.

- Upon receipt of a written application for a main extension, the Company will provide the applicant(s) an itemized estimate of the cost of the proposed extension. Said estimate shall include the cost of all labor and materials required, including valves, fire hydrants, booster stations, storage facilities, reconstruction of existing mains (if necessary), and the direct costs associated with supervision, engineering, permits, and bookkeeping.  The estimate will not include unanticipated costs such as rock excavation.

- Applicant(s) shall enter into a contract with the Company for the installation of said extension and shall tender to the Company the amount determined in paragraph B. above.  Any applicable New Service Connection Fee will become due after the cost incurred by the Company has been ascertained, as per Rule 5 B. 1. or 3., and as specified in the Schedule of Service Charges.  The contract may allow the Customer to contract with an independent contractor for the installation and supply of material, except that mains of twelve inches (12”) or greater diameter must be installed by the Company, and the reconstruction of existing facilities must be done by the Company.

- The cost to single-family residential applicant(s) connecting to a main extension for which  other applicant(s) paid an amount determined in paragraph B., above, subject to subsequent adjustments for actual cost, shall be as follows:

  - For single-family residential applicant(s) applying for service in a platted subdivision, the Company shall divide the actual cost of the extension by the number of lots abutting said extension to determine the per lot extension cost. When counting lots, corner lots which abut existing mains shall be excluded.

  - For single-family residential applicant(s) applying for service in areas that are unplatted in subdivision lots, an applicant(s) cost shall be equal to the total cost of the main extension divided by the total length of the main extension in feet times one hundred (100) feet.

  - For industrial, commercial, or multifamily residential applicants, the cost will be equal to the amount calculated for a single-family residence in paragraphs D.1. or D.2. above, multiplied by the flow factors of the applicants' meter.  The flow factors of the various sizes of meters are as follows:

Meter Size			Flow Factor

5/8”			      1

1”			    2.5

1 ½”			     5

2”			     8

3”			    15

4”			    25

- Refunds of funds paid by applicant(s) for any estimated costs or actual costs of a main extension shall be made to such applicant(s) as follows:

  - Should the actual cost of the extension be less than the estimated cost, the Company shall refund the difference to the applicant(s) as soon as the actual cost has been ascertained.

  - During the first ten (10) years after the main extension is completed, the Company will refund to the applicant(s) who paid for the extension the money collected from applicant(s) in accordance with paragraph D. above.  The refund shall be paid within a reasonable time after the money is collected.

  - The sum of all refunds to any applicant shall not exceed the total amount which the applicant(s) has paid.

- Extensions made under this rule shall be and remain the property of the Company.

- The Company reserves the right to further extend the main and to connect mains on intersecting streets and easements.  Connecting new Customers to such further extensions shall not entitle the applicant(s) paying for the original extension to a refund for the connection of such Customers.

- Extensions made under this rule shall be of Company-approved pipe sized to meet water service requirements.  If the Company chooses to size the extension larger in order to meet the Company's overall system requirements, the additional cost caused by the larger size of pipe shall be borne by the Company.

- No interest will be paid by the Company of payments for the extension made by the applicant(s).

- If extensions are required on private roads, streets, through private property, or on private property adjacent to public right-of-way, a proper deed of easement must be furnished to the Company without cost to the Company, before the extension will be made.

Rule 15 – Deposits

- The Company may require a deposit as a condition of new residential service if –

  - The applicant has a past-due bill, which accrued within the last five (5) years and, at the time of the request for service, remains unpaid and not in dispute with a utility for the provision of the same type of service;

  - The applicant has, in an unauthorized manner, within the last five (5) years prior to applying for service, interfered with or diverted the service of a utility in the provision of the same type of service; or

  - The applicant’s Equifax Advanced Energy Risk Score (EAER Score) is 699 or below. If the applicant has insufficient credit history to determine a credit score, then the applicant shall be deemed to have established an acceptable credit rating if the customer meets any of the following criteria:

    - Owns or is purchasing a home;

    - Is and has been regularly employed on a full-time basis for at least one (1) year;

    - Has a regular source of income; or

    - Can provide adequate credit references from a commercial credit source.

- The Company may require a deposit as a condition of continuing or re-establishing residential service if—

            - The service of the customer has been discontinued by the utility for nonpayment of a delinquent account not in dispute; or

            - The customer has interfered with, diverted or, in an unauthorized manner, used utility service delivered to the customer’s premises; or;

            - The customer has failed to pay an undisputed bill on or before the delinquent date for five (5) billing periods out of twelve (12) consecutive monthly billing periods, or two (2) quarters out of four (4) consecutive quarters. Prior to requiring a customer to post a deposit under this subsection, the Company shall send the customer a written notice explaining the utility’s right to require a deposit or include such explanation with each written discontinuance notice.

- A deposit shall be subject to the following terms:

  - It shall not exceed four (4) times the average bill for utility charges actually incurred by the customer during the most proximate twelve (12)-month period at the service location or, in the case of a new customer, who is assessed a deposit under this rule, one-sixth (1/6) of the estimated annual bill for monthly billed customers for utility charges at the requested service location;

  - It shall bear interest at the rate specified below, which shall be credited annually to the account of the customer or paid upon the return of the deposit to the customer, whichever occurs first. Interest shall not accrue on any deposit after the date on which a reasonable effort has been made to return it to the customer. The Company shall make all reasonable efforts to return a deposit to its customer when the customer is entitled to the return of their deposit and shall keep records of efforts to return a deposit. This rule shall not preclude a utility from crediting interest to each service account during one (1) billing cycle annually;

  - Upon discontinuance or termination other than for a change of service address, it shall be credited, with accrued interest, to the Company’s charges stated on the final bill and the balance, if any, shall be returned to the customer within twenty-one (21) days of the rendition of the final bill;

  - Upon satisfactory payment of all undisputed utility charges during the last twelve (12) billing months, it shall be promptly refunded or credited, with accrued interest, against charges stated on subsequent bills. Payment of a charge is satisfactory if received prior to the date upon which the charge becomes delinquent provided it is not in dispute. Payment of a disputed bill shall be satisfactory if made within ten (10) days of resolution or withdrawal of the dispute. The Company may withhold refund of a deposit pending the resolution of a dispute with respect to charges secured by the deposit;

  - Each customer posting a security deposit shall receive, in writing, at the time of tender of deposit or with the first bill a receipt as evidence of deposit, unless the Company shows the existence or nonexistence of a deposit on the customer’s bill, in which event the receipt shall not be required unless requested by the customer. The receipt shall contain the following minimum information:

              - Name of customer;

              - Date of payment;

              - Amount of payment;

              - Identifiable name, signature, and title of the utility employee who received the payment; and

              - Statement of the terms and conditions governing the payment, retention, and return of deposits.

  - The Company shall not deprive a customer of a deposit return within five (5) years following the date that the customer is due for a deposit return, even though the customer may be unable to produce the original receipt; provided that the customer can produce adequate identification;

  - No deposit or guarantee or additional deposit or guarantee shall be required by the Company because of race, sex, creed, national origin, marital status, age, number of dependents, source of income, disability, or geographical area of residence; and

  - The Company shall permit an applicant or customer required to make a deposit to pay the deposit in installments unless the utility can show –

              - Applicant has in an unauthorized manner, interfered with, or diverted the same type of service within the last five (5) years; or

              - If a customer has in an unauthorized manner interfered with, diverted, or used the service of the utility situated on or about or delivered to the customer’s premises; or

              - A likelihood that the customer does not intend to pay for the service.