ARTICLE XI:  E911

TERMS AND CONDITIONS FOR PROVIDING

INTERCONNECTION AND DATABASE ACCESS FOR E911

This Article XI:  E911 sets forth the terms and conditions under which CenturyTel and Socket will interconnect in order to provide the connection between Socket’s local switch and CenturyTel E911 Universal Emergency Number Service customer(s).  The parties recognize and agree that CenturyTel must provide Socket with non-discriminatory access to 911 and E-911 databases on an unbundled basis, in accordance with Section 251(c)(3) of the Act.  Further, the parties recognize and agree to establish interconnection in accordance with Article V so that 911 Emergency and related calls may be routed between Socket’s customers and CenturyTel’s customers.

1.0	Definitions

As used herein and for the purposes of this Attachment the following terms will have the meanings set forth below:

  - “911 Trunk” means a trunk capable of transmitting Automatic Number Identification (ANI) associated with a call to 911 from Socket’s End Office to the E911 customer.

  - “Automatic Location Identification” or “ALI” means the automatic display at the PSAP of the caller’s telephone number, the address/location of the telephone and, in some cases, supplementary emergency services information.

1.3	“Automatic Number Identification” (ANI) or “Calling Party Number” (CPN) allows for identification of the telephone number that originates a call.  In some instances, the station number of the calling party is not identified using ANI or CPN. In these instances the Calling Party will be identified by using a billing telephone number.

1.4	“Company Identifier” or “Company ID” means a three to five (3 to 5) character identifier for by the Local Exchange Carrier that distinguishes the entity providing dial tone to the end user. The Company Identifier is maintained by NENA in a nationally accessible database.

## 1.5	“Database Management System” or “DBMS” means a system of manual procedures and computer programs used to create, store and update the data required to provide Selective Routing and/or Automatic Location Identification for 911 systems.

## 1.6	E911 Customer - A municipality or other state or local governmental unit, or an authorized agent of one or more municipalities or other state or local government units to whom authority has been lawfully delegated to respond to public emergency telephone calls, at a minimum, for emergency police and fire service through the use of one telephone number, 911.

  - “E911 Universal Emergency Number Service” (also referred to as “Expanded 911 Service” or “Enhanced 911 Service” or “E911 Service”) means a telephone exchange communications service whereby a public safety answering point (PSAP) answers telephone calls placed by dialing the number 911. E911 includes the service provided by the lines and equipment associated with the service arrangement for the answering, transferring, and dispatching of public emergency telephone calls dialed to 911. E911 provides completion of a call to 911 via dedicated trunking facilities and includes Automatic Number Identification (ANI), Automatic Location Identification (ALI), and/or Selective Routing (SR).

  - “Emergency Services” means police, fire, ambulance, rescue, and medical services.

  - “Emergency Service Number” or “ESN” means a three to five digit number representing a unique combination of emergency service agencies (Law Enforcement, Fire, and Emergency Medical Service) designated to serve a specific range of addresses within a particular geographical area.  The ESN facilitates selective routing and selective transfer, if required, to the appropriate PSAP and the dispatching of the proper service agency (ies).

  - “National Emergency Number Association” or “NENA” means a not-for-profit corporation established in 1982 to further the goal of “One Nation-One Number” for emergency calls.  NENA is a networking source and promotes research, planning, and training.  NENA strives to educate, set standards and provide certification programs, legislative representation and technical assistance for implementing and managing 911 systems.

1.11	Public Safety Answering Point (PSAP) - An answering location for 911 calls originating in a given area.  The E911 customer may designate a PSAP as primary or secondary, which refers to the order in which calls are directed for answering.  Primary PSAPs respond first, secondary PSAPs receive calls on a transfer basis only.  PSAPs are public safety agencies such as police, fire, emergency medical, etc., or a common bureau serving a group of such entities.

1.12	“Selective Routing” and “Selective Router” or “SR” means the routing and equipment used to route a call to 911 to the proper PSAP based upon the number and location of the caller.  Selective routing is controlled by an ESN, which is derived from the location of the access line from which the 911 call was placed.

1.13 	ALI Database - A database which stores information associated with end user customers’ telephone numbers.

2.0	CenturyTel RESPONSIBILTIES

2.1	When CenturyTel is the 911 Service Provider in a particular Rate Center in which Socket is authorized to provide local telephone exchange service, CenturyTel shall have the following obligations:

  - Call Routing

    - CenturyTel will route 911 calls originated by the Socket’s end-users from the applicable POI and will switch 911 calls through the SR to the designated primary PSAP or to designated alternate locations, according to routing criteria specified by the PSAP.

    - CenturyTel will forward the calling party number (ANI) it receives from Socket and the associated 911 Automatic Location Identification (ALI) to the applicable PSAP for display.  If no ANI is forwarded by Socket, CenturyTel will forward an Emergency Service Central Office (ESCO) identification code for display at the PSAP.  If ANI is forwarded by Socket, but no ALI record is found in the E911 DBMS, CenturyTel will report this “No Record Found” condition to Socket in accordance with NENA standards.

2.3.0	FACILITIES AND TRUNKING

2.3.1	CenturyTel shall provide and maintain sufficient dedicated facilities and E911 trunks from each applicable POI to the CenturyTel SR that serves a particular exchange or exchanges being served by Socket and from the CenturyTel SR to the PSAP(s) of the E911 Customer, according to rules of the Missouri Public Service Commission, and documented specifications of the E911 Customer.

2.3.2	Upon written request by Socket, CenturyTel shall, in a timely fashion and at no charge, provide Socket with a description of the geographic area (or list of Rate Centers) which it serves as the E911 service provider and a listing of PSAPs served by the E911 SR based upon the standards set forth in the May 1997 NENA Recommended Standards for Local Service Provider Interconnection Information Sharing, or any subsequent revision(s) thereto.

2.3.3	CenturyTel and Socket will cooperate to promptly test all trunks and facilities between Socket's switch and the CenturyTel SR(s)in accordance with industry standards to ensure proper functioning of 911 service.

2.3.4	Upon request, CenturyTel is responsible for providing routed diverse routing  for 911 connectivity.

2.4	Database

2.4.1	Where CenturyTel manages the E911 database, CenturyTel shall store Socket's end user 911 Records [that is, the name, address, and associated telephone number(s) for each end user served by Socket] in the electronic data processing database for the E911 DBMS.  Socket or its representative(s) is responsible for electronically providing end user 911 Records and updating this information.

2.4.2	Where CenturyTel manages the E911 database, CenturyTel shall protect access by use of a password to the E-911 database or DBMS.  The parties shall access the E-911 database or DBMS solely for the purpose of updating and managing subscriber records.

2.4.3	CenturyTel shall coordinate access to the CenturyTel E911 DBMS for the initial loading and updating of Socket end user 911 Records.

2.4.4	CenturyTel ALI database shall accept electronically transmitted files that are based upon NENA standards.  Manual entry shall be allowed only in the event that DBMS is not functioning properly.

2.4.5	CenturyTel will update Socket’s end user 911 Records in the E911 DBMS, at no charge to Socket, if Socket uses CenturyTel’s E911 Gateway to maintain Socket’s end user records.  CenturyTel will provide Socket an error and status report. Within 24 hours of updating 911 database.

2.4.6.	Where CenturyTel manages the DBMS, CenturyTel shall provide Socket with a file containing the Master Street Address Guide (MSAG) for Socket’s respective exchanges or communities.  The MSAG will be provided on a routine basis but only for those areas where Socket is authorized to do business as a local exchange service provider.  In lieu of a file and regular updates, CenturyTel may continue to provide direct electronic access to the MSAG database.

2.4.7	Where CenturyTel manages the DBMS, CenturyTel shall establish a process for the management of NPA splits by populating the DBMS with the appropriate NPA codes.

2.4.8	CenturyTel will be responsible for identifying and correcting database errors that are caused by CenturyTel.

2.4.9	Pursuant to its tariffs, CenturyTel may charge the appropriate E-911 customer for each Socket subscriber record that CenturyTel maintains in the E-911 database or DBMS.

3.0	Socket RESPONSIBILITIES

When CenturyTel is the E911 Service Provider in an exchange where Socket is authorized to provide basic local exchange service and wants to commence provision of such service, Socket shall have the following obligations:

## Call Routing

    - Socket will transport 911 calls from its switch to the applicable POI associated with each exchange where Socket provides basic local exchange service or other service that includes 911 or E-911 functionality.

    - Socket will forward the ANI information of the party calling 911 to CenturyTel via the applicable POI.

## Facilities and Trunking

    - Socket shall provide sufficient facilities and trunks on its side of the POI in order to interconnect with CenturyTel at the applicable POI to be able to route E-911 calls originated by its end-user to CenturyTel.

    - Socket shall provide a minimum of two (2) one-way outgoing E911 trunk(s) dedicated for originating 911 emergency service calls to each POI for each CenturyTel 911 Selective Router, where applicable.  Socket shall engineer its 911 trunks to attain a minimum P.01 grade of service as measured using the “busy day/busy hour” criteria or, if higher, at such other minimum grade of service as required by Applicable Law or duly authorized Governmental Authority.

    - Socket is responsible for requesting that trunking and facilities be routed diversely for 911 connectivity.

    - Socket is responsible for determining the proper quantity of trunks and facilities from its switch(es) to the POI.

    - Socket will cooperate with CenturyTel to promptly test all 911 trunks and facilities between Socket’s switches and the CenturyTel 911 Selective Router(s), in accordance with industry standards, to assure proper functioning of 911 service.

    - At its sole discretion Socket may choose to connect directly to a SR.   To reach the SR, Socket may provide its own facilities, secure facilities from a third party, lease special access facilities from CTEL or lease interconnection facilities from CTEL at rates sets forth in this agreement.

## Database

    - Once E911 trunking has been established and tested between Socket’s End Office and appropriate POIs, Socket or its representatives shall be responsible for providing Socket's end user 911 Records to CenturyTel for inclusion in CenturyTel’s DBMS on a timely basis.

    - Socket or its agent shall provide initial and ongoing updates of Socket's end user 911 Records that are MSAG-valid in electronic format based upon established NENA standards.

    - Socket shall adopt use of a Company ID on all Socket end user 911 Records in accordance with NENA standards.  The Company ID is used to identify the carrier of record in facility configurations.

    - Socket is responsible for providing CenturyTel updates to the ALI database; in addition, Socket is responsible for correcting any errors that may occur during the entry of their data to the CenturyTel 911 DBMS.

    - Socket shall be solely responsible for providing test records and conducting call-through testing on all new exchanges.

4.0	RESPONSIBILITIES OF BOTH PARTIES

  - The Parties shall jointly coordinate the provisioning of transport capacity sufficient to route originating 911 calls from Socket through the applicable POI(s) to the designated CenturyTel 911 Selective Router(s).

  - Where SS7 connectivity is available and required by the applicable E911 Customer, the Parties agree to implement Common Channel Signaling trunking rather than CAMA MF trunking.

  - Socket is responsible for the isolation, coordination and restoration of all 911 network maintenance problems on its side of POI.  CenturyTel will be responsible for the isolation, coordination and restoration of all 911 network maintenance problems on its side of the POI.  Socket is responsible for advising CenturyTel of the circuit identification and the fact that the circuit is a 911 circuit when notifying CenturyTel of a failure or outage. The Parties agree to work cooperatively and expeditiously to resolve any 911 outage. CenturyTel will refer network trouble to Socket if no defect is found in CenturyTel’s  911 network.  The Parties agree that 911 network problem resolution will be managed expeditiously at all times.

  - CenturyTel and Socket shall arrange for the automated input and periodic updating of Socket's end user 911 Records.

  - Each Party is responsible for collecting from its retail end users and remitting to the appropriate municipality or other governmental entity any applicable 911 surcharges assessed on the local service provider and/or retail end users by any municipality or other governmental entity within whose boundaries the Party provides local exchange service.

5.0	METHODS AND PRACTICES

5.1	With respect to all matters covered by this Attachment, each Party will comply with all of the following to the extent that they apply to E911 Service:  (i) all FCC and applicable state Commission rules and regulations, (ii) any requirements imposed by any Governmental Authority other than a Commission, and (iii) the principles expressed in the recommended standards published by NENA.

6.0	CONTINGENCY

6.1	The Parties agree that E911 service is provided for the use of the E911 customer, and recognize the authority of the E911 Customer to establish service specifications and grant final approval (or denial) of service configurations offered by CenturyTel and Socket.

7.0	LIABILITY

7.1	CenturyTel liability and potential damages, if any, for its gross negligence, recklessness or intentional misconduct, is not limited by any provision of this Appendix. CenturyTel shall not be liable to Socket, its End Users or its E911 calling parties or any other parties or persons for any loss arising out of its performance under this Article or the provision of E911 Service or any errors, interruptions, defects, failures or malfunctions of E911 Service, including any and all equipment and data processing systems associated therewith.  Damages arising out of such interruptions, defects, failures or malfunctions of the system after CenturyTel has been notified and has had reasonable time to repair, shall in no event exceed an amount equivalent to any charges made for the service affected for the period following notice from Socket until service is restored.

7.2	Socket’s liability and potential damages, if any, for its gross negligence, recklessness or intentional misconduct is not limited by any provision of this Article.  Socket shall not be liable to CenturyTel, its End Users or its E911 calling parties or any other parties or persons for any loss arising out of it performance under this Article or the provision of E911 Service or any errors, interruptions, defects, failures or malfunctions of E911 Service, including any and all equipment and data processing systems associated therewith.  Damages arising out of such interruptions, defects, failures or malfunctions of the system after Socket has been notified and has had reasonable time to repair, shall in no event exceed an amount equivalent to any charges made for the service affected for the period following notice from CenturyTel until service is restored.