Feature Group C Network Principles

The following principles shall be used to guide the ILEC industry in resolving Feature Group C (FGC) Network utilization and compensation.

Any ILEC that directly connects to a facilities based CLEC or CMRS provider (either at its own tandem or end office), and performs a transiting function that places traffic from that entity onto the FGC Network, will make available compensation billing records for that traffic to all carriers on the call path.

Any ILEC that provides UNE-P service to a CLEC, and performs a transiting function that places traffic from that entity onto the FGC Network, will make available compensation billing records for that traffic to all carriers on the call path.

ILEC’s that route IntraLATA toll traffic onto the FGC Network retain the obligation to participate in the Texas IntraState IntraLATA Compensation Plan (TIICP) by generating the appropriate “92 records” and distributing them in accordance with the TIICP.

Any ILEC that performs a FGC transiting function for its CLEC affiliate has the specific obligation to ensure that appropriate CLEC FGC Network compensation billing records are made available for that traffic to all carriers on the call path.  This can be handled by the CLEC creating the appropriate “92 records” or by the ILEC making available the appropriate compensation billing records.

ILEC’s that are required to make compensation billing records available for transiting traffic may generate 11-01-01 or 92-01-01 call detail records.  Each ILEC will focus on converting to 11-01-01 records as soon as economically practical.

Any ILEC that provides an IntraLATA Toll or Access wholesale service that utilizes the FGC network shall generate “92 records” compensation billing records for that traffic to all carriers on the call path in accordance with the TIICP.

The “92 records” process will not be expanded to include ILEC originated local, EMS, EAS, ELCS traffic, as it is likely that the “92 records” process will be phased out over time.

Due to the relatively small volume of Feature Group A traffic, the existing settlement process will continue.

Due to the relatively small volume of IntraLATA 800 traffic, the existing settlement process will continue.

Each ILEC shall complete the attached Feature Group C Network Compensation Billing Records Profile that describes the method by which an ILEC may obtain the appropriate FGC compensation billing records.

Feature Group C Network Compensation Billing Records Profile

ILEC Company Name: _____________________________________________

ILEC OCN:_______________________________________________________

ILEC Contact Name: _______________________________________________

ILEC Contact Address: _____________________________________________

ILEC Contact Number: _____________________________________________

ILEC Contact Email Address: ________________________________________

ILEC performs FGC Network transiting function for facilities based CLEC, UNE-P CLEC or CMRS provider.     Yes           No      (If yes, provide the following additional information.)

Describe method ILEC will obtain FGC Network transit traffic that is originated by a CMRS provider.

Describe method ILEC will obtain FGC Network transit traffic that is originated by a facilities based CLEC.

Describe method ILEC will provide FGC Network transit traffic that is originated by a UNE-P based CLEC.